This is a Beta Release, intended only for evaluation purpose. Contact your local SMSC FAE to get the latest software package.

# HAL Modifications For Intel StrongARM* SA1111 Development Board To Support LAN91C96

- The interrupt generated from SMSC LAN91C96 is shared by other devices on the SA1111DB.We have created a new system interrupt ID for LAN91C96 (SYSINTR_91C96) to handle the interrupts generated by LAN91C96.Following modifications are to be made in the HAL (Hardware Adaptation Layer) to add the support for SYSINTR_91C96.

- Existing code uses Interrupt Level 16 to interrupt but the developer can choose anything as long as it is available. To change the interrupt level, the developer has to change the definition of SYSINTR_91C96.

- In </Platform\SA11X0BD\inc> Modify OALINTR.H

- #define SYSINTR_91C96			(SYSINTR_FIRMWARE+16)

- In </Platform\SA11X0BD\inc> Modify OALINTR.INC

- SYSINTR_91C96: .equ		SYSINTR_FIRMWARE+16

- In </Platform\SA11X0BD\inc> Modify OALINTRA.INC

- SYSINTR_91C96: .equ		SYSINTR_FIRMWARE+16

- In <\Platform\SA11X0BD\Kernel\Hal> Modify OEMEnableInterrupt() in Cfwsarm.c

- case SYSINTR_91C96:

- #ifndef USING_SA11X1

- ETHER_DEBUG_INT_ALTERNATE (0);

- ETHER_DEBUG_INT_INIT;

- #endif

- ETHER_DEBUG_INT_FALLING_EDGE;   // set irqs to falling edge

- ETHER_DEBUG_INT_CLR (1);        // initial clear to irqs

- ETHER_DEBUG_INT_EN (1);         // enable irqs*/

- #ifdef USING_SA11X1

- DAUGHTER_CARD_INT_MASK (1);     // disable SA1101CB interrupts

- #else

- GPIO_GRP11_27_INT_MASK (1);

- #endif

- break;

- In <\Platform\SA11X0BD\Kernel\Hal> Modify OEMDisableInterrupt() in Cfwsarm.c

- case SYSINTR_91C96:

- ETHER_DEBUG_INT_EN (0);   // disable irqs

- break;

- In <\Platform\SA11X0BD\Kernel\Hal> Modify OEMInterruptDone() in Cfwsarm.c

- case SYSINTR_91C96:

- ETHER_DEBUG_INT_EN (1);         // enable irqs

- break;

- In <\Platform\SA11X0BD\Kernel\Hal\ARM> Modify OEMInterruptHandler() in Int11x0.c

This is TRUE only for SA1111 platform. You can do declarations and definitions with other declarations and definitions in OEMInterruptHandler() function in this file.

- #define READ_PORT_BYTE(port)         (*(volatile PBYTE const)(port))

- #define WRITE_PORT_BYTE(port, value) (*(volatile PBYTE const)(port) = value)

- volatile BYTE *pbEthernetBaseHAL;

- #define ReadWord(wOffset) \

- (   READ_PORT_BYTE((PBYTE)(pbEthernetBaseHAL + (wOffset*4)))+\

- (((USHORT)READ_PORT_BYTE((PBYTE)(pbEthernetBaseHAL + ((wOffset+1)*4))))<<8))

- #define WriteWord( wOffset, Value ) \

- {   WRITE_PORT_BYTE((PBYTE)(pbEthernetBaseHAL + (wOffset*4)),(BYTE)(Value));\

- WRITE_PORT_BYTE((PBYTE)(pbEthernetBaseHAL + ((wOffset+1)*4)),(BYTE)(Value>>8));}

- volatile struct SA1111DBRegisterBlock *v_pEnetRegsHAL;

- volatile BYTE *v_pioRegsHAL = NULL;

- v_pEnetRegsHAL=(struct SA1111DBRegisterBlock*)SA11X1DB_REG_BASE_U_VIRTUAL;

- // Getting the pointer to virtual address space for SMSC LAN91C96 IO space. PG 10/03/00

- v_pioRegsHAL = (BYTE *)SMC_ETHER_IO_BASE;

## Find if (HCI_USBPRES_INT_STAT_SNAPSHOT) and add this as another else if statement. Please see the attached file inttmp.c as a sample.

- if(v_pEnetRegsHAL->irq[0].enet == 0){

- pbEthernetBaseHAL = (unsigned)v_pioRegsHAL;

- SavedBank = ReadWord(14);

- WriteWord(14,0x3302)

- IntrSts = READ_PORT_BYTE((PBYTE)pbEthernetBaseHAL + 4*12);

- IntrMask = READ_PORT_BYTE((PBYTE)pbEthernetBaseHAL + 4*13);

- if(!IntrMask)

- {

- WriteWord( 14, SavedBank );

- v_pDCIRQReg->intstatclr0[0] = intStat0;

- v_pDCIRQReg->intstatclr1[0] = intStat1;

- retval = SYSINTR_NOP;

- }

- if((IntrSts & (ENABLED_INTS | INT_ALLOC)) && (IntrMask))

- {

- WRITE_PORT_BYTE((PBYTE)pbEthernetBaseHAL + 4*13,(UCHAR)0);

- WriteWord( 14, SavedBank );

- retval = SYSINTR_91C96;

- }

- else

- {

- WriteWord( 14, SavedBank );

- v_pDCIRQReg->intstatclr0[0] = intStat0;

- v_pDCIRQReg->intstatclr1[0] = intStat1;

- retval = SYSINTR_NOP;

- }

- } 	else{

- // if we don't recognize the interrupt, clear them and return

- v_pDCIRQReg->intstatclr0[0] = intStat0;

- v_pDCIRQReg->intstatclr1[0] = intStat1;

- retval = SYSINTR_NOP;

- }

- <\Platform\SA11X0BD\inc> Modify SA11X0BD.INC.file

- ; SA1111DB SMSC LAN91C96 Ethernet Chip io Info get from Intel

- ETHERNET_DEVICE_IO_PHYSICAL    		EQU 		0x18000000

- ETHERNET_DEVICE_IO_C_VIRTUAL		EQU 		0x8BA00000

- ETHERNET_DEVICE_IO_U_VIRTUAL		EQU (ETHERNET_DEVICE_IO_C_VIRTUAL+CACHED_2_UNCACHED_OFFSET)

- ; SA1111DB SMSC LAN91C96 Ethernet Chip attr Info get fron Intel

- ETHERNET_DEVICE_ATTR_PHYSICAL		EQU				0x1A000000   ETHERNET_DEVICE_ATTR_C_VIRTUAL	EQU 				0x8BC00000

- ETHERNET_DEVICE_ATTR_U_VIRTUAL	EQU (ETHERNET_DEVICE_ATTR_C_VIRTUAL+CACHED_2_UNCACHED_OFFSET)

- In OEMAddressTable() function in SA11x0bd.inc

- ; SA11x1Bd buildin ethernet Attr

- DCD ETHERNET_DEVICE_ATTR_C_VIRTUAL, ETHERNET_DEVICE_ATTR_PHYSICAL,1

- ; SA11x1Bd buildin ethernet IO

- DCD ETHERNET_DEVICE_IO_C_VIRTUAL, ETHERNET_DEVICE_IO_PHYSICAL, 1

- In ROMAddressTable() function in SA11x0bd.inc

- ; SA11x1Bd buildin ethernet Attr

- DCD ETHERNET_DEVICE_ATTR_C_VIRTUAL, ETHERNET_DEVICE_ATTR_PHYSICAL, 1

- ; SA11x1Bd buildin ethernet IO

- DCD ETHERNET_DEVICE_IO_C_VIRTUAL, ETHERNET_DEVICE_IO_PHYSICAL, 1

- <\Platform\SA11X0BD\inc> Modify Sa11X0BD.H file

- #define SMC_ETHER_IO_BASE              			   0xABA00000

- #define SMC_ETHER_ATTR_BASE			   0xABC00000

#define ETHERNET_DEVICE_IO_PHYSICAL     0x18000000   // SA1111DB EthernetChip io     Info get fron Intel

- #define ETHERNET_DEVICE_ATTR_PHYSICAL   0x1A000000   //; SA1111DB EthernetChip attr Info get fron Intel

- (You can always provide the different virtual base addresses for SMSC Ethernet IO and Attribute Registers that are not occupied in the system)

After doing these modifications, build new HAL (nk.exe, nknodbg.exe and nkprof.exe). and copy to $(_FLATRELEASEDIR) directory.

Note: When you are ready to build LAN9000.DLL (drivers for LAN91C96), you should have done all modifications described above because there are some dependencies in the driver source code. If you don’t do the recommended modifications, you may encounter compilation errors or drivers may not work properly. To build the drivers for LAN91C96, always follow the BUILD.DOC provided with the software.

- Please read the Intel StrongARM SA-111x documents to understand the hardware portion of LAN91C96. You can download the datasheet for LAN91C96 from SMSC website (www.smsc.com)