This is a Beta Release, intended only for evaluation purpose. Contact your local SMSC FAE to get the latest software package.

- SMSC LAN9000 NDIS 4.0

- Miniport Windows CE 3.0 Device Driver for

- Intel StrongARM* SA-111x Development Platform Release Note Version 1.0

## Summary

This note is explicitly for OEMs. This note contains the software information about the LAN9000 products that involve 91C96 in respect of NDIS 4.0 Miniport Windows CE Device Drivers for Intel StrongARM* SA-1111 Platform with SA 1110 CPU and SA 1111 Companion Chip. The OEMs can use this note for developing and testing the customized drivers.

## Introduction

The SMSC LAN9000 NDIS 4.0 Windows CE device drivers are intended for SMSC LAN9000 chips. The driver currently supports 91C96 in PCMCIA mode. The drivers support Windows CE 3.0 operating systems. The Single function device has a network adapter that is supported by the drivers provided by SMSC.

Supported Features

Processors 			:	StrongARM*  SA-1110  and SA-1111

Bus Type			:	PCMCIA.

It has following characteristics:

- Support for StrongARM* SA-1110 and SA-1111 processor.

- Support for PCMCIA bus..

- Tests passed: PING and SLEDGE.

- Drivers are built into the CE OS and it should be loaded using EBOOT program or other Windows CE image loading program provided with Platform Builder on Intel StrongARM* SA-1111 Reference Platform.

## Package Contents

- LAN9000.DLL: LAN9000 Driver file

- PLAT.REG

- PLAT.BIB

- Set.bat

- Release.doc :  Release Note

- Build.doc describes the procedure to build the customized CE image for StrongARM* SA-111x.

- LAN91C96-SA1111DB.doc

Creating Windows CE Image with LAN9000 Drivers

Please go through the Build.Doc and LAN91C96-SA1111DB included in this package to understand the complete build process of Windows CE image with LAN9000 Drivers. Please refer to the ETK/Platform Builder Documentation for further assistance.

## Installation

### Installation on Windows CE 3.0

- Hook up LAN91C96 on the Intel StrongARM* SA-1111 Reference Platform to 10 Mbps network.

- Load Windows CE 3.0 image with LAN9000 drivers using EBOOT procedure. Please read Windows CE Platform Builder Documentation to understand the Ethernet bootloader procedure to download the Windows CE image.

- Please read the Intel StrongARM* SA-1110 Development Platform Windows CE Software-User Guide to build the Windows CE image with SA11X0BD platform.

It will load the Windows CE image on Intel StrongARM SA-1111 Platform. You can use other ways of installing the CE images, provided with Windows CE 3.0.

## Tested Configuration / Performance

Way to verify that the drivers working such as:

- PING

- You need a Windows NT Server 4.0 to test the functionality of the adapter. You need to run a DHCP server.

- Follow the procedure to test the functionality of the drivers

- Configure and load DHCP server on NT machine.

- Verify that the Test LAN91C96 is connected to the same network.

- Load the newly created CE image (NK.BIN) on target machine as described above.

- Setup the DHCP configuration on Network Adapter on a test platform. It has been setup by default. So you don’t need to any modifications.

- Open a Command window (CMD.EXE) on the Windows NT 4.0 server and ping the test Intel StrongARM* SA-1111 Platform. Click the Windows Start button and choose Run.

- Type “Ping” plus the IP Address to the CEPC test platform in the Open box. For example, if the IP address assigned to the Intel StrongARM SA-1111 Platform is 192.101.55.102, then enter the following command at the command line:

- Ping 192.101.55.102

- Your output should be similar to the following:

- Pinging 192.101.55.102 with 32 bytes of data:

- Reply from 192.101.55.102: bytes=32 time=10ms TTL=32

- Reply from 192.101.55.102: bytes=32 time=10ms TTL=32

- Reply from 192.101.55.102: bytes=32 time=10ms TTL=32

- Reply from 192.101.55.102: bytes=32 time=10ms TTL=32