- Building Customized Image of Windows CE Operating System

- BUILD-NOTE

- Follow the steps to build a new Windows CE customized image

- Build MAXALL-built of Windows CE operating system with default configuration.

- Build MAXALL built of Windows CE image provided by ETK without any modifications.

- Attaching LAN9000 Drivers to the default built like MAXALL of Windows CE Operating System

Place Lan9000 driver file (Lan9000.dll) in Windows CE Release (_FLATRELEASEDIR) Directory.

LAN9000.DLL is only required file for LAN9000 drivers. This file is present in CE9000.ZIP file.

- FILE MODIFICATIONS

- Edit PLATFORM.BIB file at the location PLATFORM\<PROJECTPLATFORMNAME>\FILES.

- If SMSC_LAN9000

- lan9000.dll	    $(_FLATRELEASEDIR)\lan9000.dll              NK  SH

- ENDIF

2	Insert the following line of text into the .bat file in project tree location \PLATFORM>\<PROJECTPLATFORMNAME>

- Set CEPC_DDI_CT655X=1 if target machine has CT655X chipset.

- Set CEPC_DDI_S3VIRGE=1 if target machine has accelerated S3ViRGE chipset.

- Set CEPC_DDI_VGA8BPP=1 if target machine has ET4000 chipset which has been setup to the CE image included in this package.

- You can select any one of these at one time.

3.	If a directory with the following path does not exist, then create it.  (Make certain that the folder “developr” is spelled this way.)

\Wince211\developr\<User>\

If this directory does not already contain a file named “setenv.bat”, copy it from location \Public\Common\Oak\Misc to the above location and modify it as:

- set SMSC_LAN9000 =1

- REGISTRY MODIFICATIONS FOR ISA CARD

- Modify the Registry file (Platform.reg) to add the support for ISA adapter. See Below for registry modifications.

- ;Begin LAN9000 ISA Support

- [HKEY_LOCAL_MACHINE\Comm\LAN9000]

- “DisplayName”=”LAN9000 Network Adapter”

- “Group”=”NDIS”

- “ImagePath”=”lan9000.dll”

- [HKEY_LOCAL_MACHINE\Comm\LAN9000\Linkage]

- “Route”=multi_sz:”LAN90001”

- [HKEY_LOCAL_MACHINE\Comm\LAN90001]

- “DisplayName”=”LAN9000 Compatible Network Adapter”

- “Group”=”NDIS”

- “ImagePath”=”lan9000.dll”

- [HKEY_LOCAL_MACHINE\Comm\Ident]

- “Password”=”Password”

- “Domain”=”Domain”

- “UserName”=”UserName”

- [HKEY_LOCAL_MACHINE\Comm\LAN90001\Parms]

- “"BusNumber"=dword:0

- "BusType"=dword:1

- "InterruptNumber"=dword:0a

- "IoBaseAddress"=dword:0300

- "Transceiver"=dword:0

- "Duplex"=dword:0ff

- "EarlyTransmit"=dword:0

- "Pcmcia"=dword:0

- "ReceiveTimer"=dword:0

- "Speed"=dword:0ff

- [HKEY_LOCAL_MACHINE\Comm\LAN90001\Parms\TcpIp]

- "EnableDHCP"=dword:1

- "DefaultGateway"=""

- "UseZeroBroadcast"=dword:0

- "IpAddress"="0.0.0.0"

- "Subnetmask"="0.0.0.0"

- [HKEY_LOCAL_MACHINE\Comm\TcpIp\Linkage]

- “Bind”=multi_sz:”ppp”,”lan90001”

- [HKEY_LOCAL_MACHINE\Drivers\PCMCIA\LAN9000]

- /////*Comment every single entry in this section to disable the PCMCIA entry for LAN9000.

- REGISTRY MODIFICATIONS FOR PCMCIA CARD

- Modify the Registry file (Platform.reg) to add the support for PCMCIA adapter. See Below for registry modifications.

- ;Begin LAN9000 PCMCIA Support

- [HKEY_LOCAL_MACHINE\Drivers\PCMCIA\Detect\60]

- "Dll"="LAN9000.DLL"

- "Entry"="DetectLAN9000"

- [HKEY_LOCAL_MACHINE\Drivers\PCMCIA\LAN9000]

- "Dll"="ndis.dll"

- "Prefix"="NDS"

- "Miniport"="LAN9000"

- [HKEY_LOCAL_MACHINE\Comm\LAN9000]

- "DisplayName"="LAN9000 Network Adapter"

- "Group"="NDIS"

- "ImagePath"="LAN9000.dll"

- [HKEY_LOCAL_MACHINE\Comm\LAN9000\Linkage]

- "Route"=multi_sz:"LAN90001"

- [HKEY_LOCAL_MACHINE\Comm\LAN90001]

- "DisplayName"="LAN9000 Network Compatible Adapter"

- "Group"="NDIS"

- "ImagePath"="LAN9000.dll"

- [HKEY_LOCAL_MACHINE\Comm\Ident]

- "Password"="Password"

- "Domain"="Domain"

- "UserName"="UserName"

- [HKEY_LOCAL_MACHINE\Comm\LAN90001\Parms]

- "BusNumber"=dword:0

- "BusType"=dword:8

- "InterruptNumber"=dword:03

- "IoBaseAddress"=dword:0340

- "Transceiver"=dword:0

- "Duplex"=dword:0ff

- "EarlyTransmit"=dword:0

- "Pcmcia"=dword:1

- "ReceiveTimer"=dword:0

- "Speed"=dword:0ff

- [HKEY_LOCAL_MACHINE\Comm\LAN90001\Parms\TcpIp]

- "EnableDHCP"=dword:1

- ; This should be MULTI_SZ

- "DefaultGateway"=""

- ; Use zero for broadcast address? (or 255.255.255.255)

- "UseZeroBroadcast"=dword:0

- ; Thus should be MULTI_SZ, the IP address list

- "IpAddress"="0.0.0.0"

- ; This should be MULTI_SZ, the subnet masks for the above IP addresses

- "Subnetmask"="0.0.0.0"

- [HKEY_LOCAL_MACHINE\Comm\Tcpip\Linkage]

- ; This should be MULTI_SZ

- ; This is the list of llip drivers to load

- "Bind"=multi_sz:"ppp","LAN90001"

- NOTE: YOU CAN CHOOSE ANY ONE OF THE ABOVE COMBINATIONS OF THE MODIFICATIONS IN THE PLATFORM.REG FILE.

- Build CE image:

After doing the required modifications, you can build a new CE image using WinCE.BAT.