- SMSC LAN9000

- NDIS 4.0

- Miniport CE Device Driver

- Release Note

- Version 1.30

## Summary

This note is explicitly for OEMs. This note contains the software information about the LAN9000 products that involve 91C94 and 91C96 in respect of NDIS 4.0 Miniport Windows CE Device Drivers. The OEMs can use this note for developing and testing the customized drivers.

## Introduction

The SMSC LAN9000 NDIS 4.0 Windows CE device drivers are intended for SMSC LAN9000 chips. The driver currently supports 91C94, 91C96 and 91C110 in ISA mode but only 91C94 and 91C96 in PCMCIA mode. We will add the support of other LAN9000 chips in future releases. The drivers support Windows CE 2.0 and 2.1x operating systems. The Single function device has a network adapter that is supported by the drivers provided by SMSC.

Supported Features

Processors 			:	x86

Bus Type			:	ISA and PCMCIA.

Currently, LAN9000 Miniport CE Device Driver has no support for SH3, MIPS processors. This is third release of LAN9000 CE drivers.

It has following characteristics:

- Support for x86 processor.

- Support for ISA and PCMCIA bus type cards.

- Tests passed: PING and SLEDGE.

- Drivers are built into the CE OS and it should be loaded using LOADCEPC program or other Windows CE image loading program provided with ETK on a DOS based x86 machine.

- Works with Every Video adapter supported by Windows CE.

## Package Contents

- LAN9000.DLL: LAN9000 Driver file

- PLATFORM.REG

- PLATFORM.BIB

- PLAT.BAT

- SETENV.BAT

- Release.doc :  Release Note

- Build.doc describes the procedure to build the customized CE image.

Creating Windows CE Image with LAN9000 Drivers

Please go through the Build.Doc included in this package to understand the complete build process of Windows CE image with LAN9000 Drivers. Please refer to the ETK/Platform Builder Documentation for further assistance.

## Installation

### Installation on Windows CE 2.x

1.    Plug the LAN9000 board on CEPC and hook up to network.

2.    Boot a DOS x86 machine (Using a DOS boot-up floopy diskette)

- Copy the NK.BIN to C:\CE9000.

- Copy the Windows CE image loading  program(like Loadcepc ) to C:\CE9000

- Go to C:\CE9000

- Type

Loadcepc /d: 2 nk.bin

It will load the Windows CE image on x86 machine. You can use other ways of installing the CE images also provided with Windows CE 2.1x.

## Tested Configuration / Performance

Way to verify that the drivers working such as:

- PING

- You need a Windows NT Server 4.0 to test the functionality of the adapter. You need to run a DHCP server.

- Follow the procedure to test the functionality of the drivers

- Configure and load DHCP server on NT machine.

- Verify that a Test Network adapter is inserted into the test platform’s ISA slot and connected to the same network.

- Boot the test DOS based x86 machine.

- Load the newly created CE image (NK.BIN) on target machine as described above.

- Setup the DHCP configuration on Network Adapter on a test platform. It has been setup by default. So you don’t need to any modifications.

- Open a Command window (CMD.EXE) on the Windows NT 4.0 server and ping the test CEPC. Click the Windows Start button and choose Run.

- Type “Ping” plus the IP Address to the CEPC test platform in the Open box. For example, if the IP address assigned to the CEPC is 192.101.55.102, then enter the following command at the command line:

- Ping 192.101.55.102

- Your output should be similar to the following:

- Pinging 192.101.55.102 with 32 bytes of data:

- Reply from 192.101.55.102: bytes=32 time=188ms TTL=32

- Reply from 192.101.55.102: bytes=32 time=188ms TTL=32

- Reply from 192.101.55.102: bytes=32 time=188ms TTL=32

- Reply from 192.101.55.102: bytes=32 time=188ms TTL=32